# Orbital CDN — The Complete Idea

Live: orbital-cdn.vercel.app · Repo: github.com/uditjainstjis/orbital-cdn Stack: Vite · globe.gl · three.js · satellite.js — static site, no backend

## 30-second summary

Two thirds of the planet is far from a cloud region. Content served from orbit can beat long-haul fibre for those users — but only if the router respects constraints that do not exist on the ground: data centres go dark in eclipse, radiation corrupts packets over the South Atlantic Anomaly, and rain closes ground gateways.

Orbital CDN is a routing simulator for that problem. It propagates 180 satellites from real orbital elements, chooses an orbital data centre and a ground gateway under one of four policies, computes end-to-end latency from the actual geometry of the chosen path, logs the outcome, and learns from the log to route the next request better.

The finding it produces is measured, not asserted:

Orbital routing beats terrestrial fibre for 69% of requests overall — 100% for São Paulo, Lagos and Dubai, and 0% for London and New York. The advantage is concentrated exactly where cloud regions are far away, and it disappears where they are close.

## 1 · The problem

Why a space CDN is a real question and not a novelty.

Terrestrial content delivery works by putting servers physically near users. That model has a floor: it requires somewhere to put the server, power to run it, and fibre to reach it. Large parts of the world — much of Africa, South America, South and Southeast Asia, Oceania — sit thousands of kilometres from the nearest hyperscale region, and their traffic makes a long fibre journey to be served.

Light travels 47% faster in vacuum than in silica. A path that goes up to orbit, crosses several thousand kilometres by laser, and comes back down can therefore beat a fibre path over the same endpoints — if the distance is long enough to pay back the climb.

The trade is geometric, and it has a crossover distance. Below it, fibre wins because the climb dominates. Above it, vacuum wins. Published work puts that crossover at roughly 4,472 km for a 550 km shell. Everything interesting about this product lives around that number.

## 2 · What makes orbital routing different

Three constraints with no terrestrial equivalent, all of which this simulator models.

Eclipse. An orbital data centre in Earth’s shadow runs on stored charge rather than live solar. It still serves requests, but a routing policy that values sustainability should prefer a sunlit one — and the sunlit set changes continuously as the constellation moves.

Radiation. The inner Van Allen belt dips to roughly 200 km over the South Atlantic, forming the South Atlantic Anomaly. Spacecraft crossing it experience single-event upsets; real missions safe their instruments during passage — one science satellite spends about 15% of its time inside it. Published extent at ~500 km altitude is 50°S–0°, 90°W–40°E.

Weather at the ground station. Ka-band downlinks suffer rain fade. A gateway under a front is degraded for hours, then recovers. This is the one constraint that is genuinely unpredictable from orbit, which — as Section 6 explains — is what makes it the only place where learning from history adds real information.

## 3 · Architecture

origin city
   │  uplink, slant range to a satellite above 25° elevation
   ▼
LEO relay satellite  (550 km, 53° inclination)
   │  inter-satellite laser crosslinks, arcs at orbital radius
   ▼
orbital data centre  (640 km)  ── compute happens here
   │  crosslink to a satellite above the gateway
   ▼
relay satellite over the ground station
   │  downlink, slant range
   ▼
ground gateway  ── egress

Constellation. 180 satellites, 9 planes × 20, 53° inclination, 550 km — a 1/8-scale model of Starlink’s authorised Gen1 shell 1 (1,584 satellites, 72 × 22, same altitude and inclination). The scale reduction is a rendering constraint: 1,584 animated meshes will not hold 60 fps in a browser.

Propagation. Real Starlink two-line elements are fetched from CelesTrak and propagated with SGP4, with proper ECI-to-geodetic conversion through Greenwich Mean Sidereal Time. When the network is unavailable it falls back to an analytic Walker-Delta model that solves latitude from inclination and true anomaly. Satellites are attitude-aligned to their velocity vector each frame via an orbital basis.

Four orbital data centres at 640 km, distinguished from relays by having compute.

Eight ground gateways at real teleport coordinates, not city-centre pins — Seletar, Usingen, Boydton, Hitachinaka, Santana de Parnaíba, Belrose, Lekki. Real ground stations sit 35–250 km outside the cities they serve, and in a model that turns on path geometry that displacement matters. Mumbai has no published coordinate and is labelled approximate.

## 4 · How a route is chosen

The cost function is the product.

Both selection steps minimise a cost expressed entirely in latency-equivalent milliseconds, so a policy weight trades like against like:

C(DC) = w_lat · reach_ms
      + w_sol · 25 · eclipsed
      + w_rad · 40 · in_SAA
      + w_lat · 30 · gain · learned_tail_latency
      + w_rad · 30 · gain · learned_SAA_exposure

C(GW) = w_lat · reach_ms
      + w_wx  · rain_fade_ms          (0 clear · 8 cloudy · 22 rain)
      + w_wx  · 30 · gain · learned_rain_penalty

Why the unit discipline matters: an earlier version mixed a normalised distance with binary 0-or-1 penalties. A single eclipse flag then outweighed half a planet of detour, and the balanced policy would route a Delhi request to the far side of the Earth to find sunlight. Expressing eclipse as “worth 25 ms of detour at full weight” makes the trade explicit and auditable.

Latency comes from the geometry of the path actually chosen — slant ranges at the ends, great-circle arcs at orbital radius in between, and a relay to a satellite above the gateway so the downlink is a real line of sight rather than a straight line through the planet.

### The four policies

| Policy | Weights (lat / sol / rad / wx) | p50 RTT | Rain exposure | Character |
| --- | --- | --- | --- | --- |
| latency | 0.95 / 0.05 / 0.05 / 0.05 | 61 ms | 13.7% | shortest path, ignores everything else |
| balanced | 0.50 / 0.50 / 0.50 / 0.40 | 67 ms | 13.9% | moderate hedging |
| reliable | 0.20 / 0.30 / 0.95 / 0.90 | 73 ms | 0.0% | accepts longer paths for zero weather exposure |
| green | 0.20 / 0.90 / 0.20 / 0.30 | 78 ms | 5.5% | maximises solar-served compute |

This is a genuine Pareto frontier, and it emerged rather than being designed. Reliable achieving exactly zero rain exposure for a 12 ms median cost is the clearest example — nobody specified it; it is what a 0.90 weather weight does.

## 5 · Product state

An append-only event log — src/telemetry.js. Every request the network serves writes one flat record: timestamp, origin, service, policy, data centre, gateway, eclipse and SAA flags, hop count, RTT, fibre baseline, and the propagation / compute / weather components separately.

Held in localStorage, capped at 6,000 records, with quota-exhaustion recovery that drops the oldest half and retries. There is no backend by design — the product is a static site — but the shape is a plain event log, so moving to a real time-series store is a transport change rather than a rewrite.

Seeded history. The network ships with 30 days of its own operating history, so the analytics view is populated on a cold open rather than empty for anyone who has not generated traffic themselves. It is generated by the same path geometry, the same fibre baselines and the same cost functions the live engine uses — not a parallel implementation — driven by three time-varying inputs: diurnal demand per city, orbital eclipse against sub-solar longitude, and weather fronts that persist for hours. The generator is deterministically seeded, so every viewer sees identical history.

This is labelled SIMULATED in the interface, with the seed shown. The alternative — letting a viewer discover the synthetic flag in the exported JSON themselves — converts a normal property of a simulator into a credibility problem.

## 6 · Adaptation

The distinction that matters: a summary that changes behaviour, versus a summary that renders charts.

The selected analysis window is also the learning window. Aggregates over it become routing biases applied to the next request.

What is learned, and why each choice is defensible:

| Signal | Source | Why it is real information |
| --- | --- | --- |
| Gateway rain penalty | observed weather-induced delay per gateway | weather is genuinely unpredictable from orbit; history is the only way to know a site is persistently bad |
| DC tail-latency penalty | observed p95 excess per data centre | persistent geometric disadvantage that no instantaneous snapshot reveals |
| DC radiation penalty | observed SAA-transit share per data centre | accumulated exposure, weighted by the radiation coefficient |
| Best policy per origin | empirical p95 by policy | surfaced as a recommendation, deliberately not auto-applied |

What is deliberately not learned: historical eclipse share. The engine already reads the eclipse flag instantaneously, so a windowed average of it carries strictly less information than the value one line above it in the same function — and charging for both double-counts the same orbital geometry.

This is the sharpest test of whether an adaptive loop is honest. If a system “learns” something it can already observe directly, the learning is decorative. Weather passes the test because it is unpredictable; eclipse fails it because it is deterministic orbital mechanics.

Measured effect:

- 6 of 32 origin/policy combinations route differently with adaptation on

- Mean RTT 115 ms adaptive versus 118 ms fixed, with no degenerate routes

- 4 of 32 decisions differ between the 7D and 30D windows; 5 of 32 between 24H and 7D

Honest limitation: the data-centre half of the loop is weak. Averaged over days, every data centre spends a similar fraction of its orbit in eclipse, so there is little to learn there. That is a property of the physics, not a tuning choice, and the panel shows the learned values as they are rather than inflating a gain to make the effect appear larger.

## 7 · What the user sees

At load, no interaction: the right-hand panel carries the window’s win rate against fibre and the mean saving, p50/p95/solar, per-origin win bars split into where orbital wins and where it loses, and the fastest policy actually observed. The topbar badge reads the finding — 69% BEAT FIBRE · 68ms p50 — rather than a record count.

On sending a request: an eight-beat camera sequence with the reasoning surfaced step by step — origin lock, uplink selection, data-centre choice with the winning cost in milliseconds against the alternatives, inter-satellite traversal, compute, gateway choice, an adaptation card naming what history contributed, and the result with a real per-policy comparison.

In the analytics view: window selector, six KPIs, a traffic-and-latency chart with clipped edge buckets marked, four categorical breakdowns, the grounding table, the where-orbital-wins panel, the adaptation panel with an on/off toggle, an adaptive-versus-fixed comparison gated at eight samples per side, and JSON export.

In Architecture & Insights: the model, the implemented cost function, the orbital mechanics, the space environment, and references.

## 8 · Grounding

The criticism that mattered most was that every number came from our own simulator. The central comparison no longer does.

The terrestrial baseline is measured: ICMP RTT between WonderNetwork VPS nodes, cross-checked against Microsoft’s published 30-day P50 backbone medians. Datacentre-to-datacentre, therefore excluding consumer last-mile — which makes it conservative, understating rather than inflating the orbital advantage. The physics model is retained as a fallback and displayed alongside, so where our prediction disagrees with reality the gap is visible rather than hidden.

External validation. Published crossover for a 550 km shell is 4,472 km against idealised fibre, shortening against realistically routed fibre; earlier work finds the same effect above ~3,000 km for a higher shell. Using measured baselines this simulator loses at London (637 km) and New York (325 km) and wins from Delhi (4,145 km) upward — the same crossover region, arrived at independently.

Constants checked against published sources: the refractive index of silica is exact; the route-winding factor of 1.42 is optimistic on its own (published rule of thumb is 2.1, best-achievable 1.3), but the composite model lands at an effective stretch of 1.84–1.96 for long haul against a measured median of 2.05.

## 9 · Engineering worth noting

Rendering. A custom GLSL shader material for city lights masked by real sun direction, a fresnel atmosphere, and a sun vector computed from UTC hour angle and seasonal declination. Pre-allocated temporary vectors avoid garbage-collection pressure in the 60 fps loop; the inter-satellite mesh is a single buffer with an adjusted draw range rather than per-edge objects; antimeridian wrapping is guarded.

Robustness lessons, each of which was a real failure. A sleep helper must resolve its promise when cleared, not merely cancel the timer — cancelling alone left the sequence suspended forever and permanently disabled the interface. requestAnimationFrame never fires in a background tab, so awaiting one unraced meant the application silently never booted for anyone who opened the link in a background tab.

## 10 · Where this goes

Nearer term. Replay real backdated two-line elements so satellite positions in the history are genuine and only demand is synthetic — closing the last gap in Section 5. Model the 5,400 km maximum inter-satellite laser range as a real routing constraint. Widen the measured-baseline set beyond eight origins.

Further out. The same multi-objective machinery applies directly to Earth-observation downlink and contact scheduling, a problem with real constraints, real operators and a real incumbent baseline today. Every input there is externally verifiable and every output is falsifiable against greedy contact scheduling. Orbital compute is the speculative setting; the scheduling core is not.

## What we are not claiming

- The 30-day history is simulated and labelled; grounding fixes the baseline it is compared against, not the traffic itself.

- The data-centre feedback loop is weak for a physical reason and is reported as such.

- The constellation is a 1/8-scale model of the shell it imitates.

- The South Atlantic Anomaly is modelled as a rectangle over an oval that has split into two lobes and drifts west; the published extent is used and over-includes the corners.

- Maximum inter-satellite link range is not modelled, which at this scale is a real omission.

Detail on any section — measurement sources, the cost-function derivation, the full defect list — available on request.