# Orbital CDN — Complete Technical Reference

Live: orbital-cdn.vercel.app · Repo: github.com/uditjainstjis/orbital-cdn Stack: Vite · globe.gl · three.js · satellite.js — static site, no backend Offline toolchain: Python 3 · NumPy · pandas · scikit-learn · LightGBM

## 30-second summary

Orbital CDN routes content requests through a simulated LEO constellation, choosing an orbital data centre and a ground gateway under a multi-objective cost function, then learns from what it served and predicts what the network will do next.

Three properties distinguish it from a visualisation:

- The physics is a standards implementation. ITU-R P.838-3, P.618-13, P.839-4, unit-tested against all 28 published coefficients.

- The comparison is externally checkable. Fibre baselines are measured third-party RTTs; the weather is real NASA POWER observations.

- The claims are falsifiable and some of them are negative. The headline experiment found that most of the benefit comes from observing, not forecasting — the opposite of what a predictive system is usually sold on.

# PART I · THE PROBLEM

## 1 · Why a space CDN is a real question

Terrestrial content delivery puts servers physically near users. That requires somewhere to put them, power, and fibre. Large regions — much of Africa, South America, South and Southeast Asia, Oceania — sit thousands of kilometres from the nearest hyperscale region.

Light travels 47% faster in vacuum than in silica. A path going to orbit, crossing by laser, and returning can beat fibre over the same endpoints if the distance is long enough to repay the climb.

The trade is geometric and has a crossover distance. Published work puts it at 4,472 km for a 550 km shell.

## 2 · Constraints with no terrestrial equivalent

| Constraint | Physical basis | Modelled as |
| --- | --- | --- |
| Eclipse | A DC in Earth’s shadow runs on stored charge | Sub-solar longitude test per satellite per frame |
| Radiation | Inner Van Allen belt dips to ~200 km over the South Atlantic | Bounding box 50°S–0°, 90°W–40°E |
| Rain fade | Ka-band downlinks attenuate in precipitation | Full ITU-R chain, Part III |

# PART II · ORBITAL MECHANICS AND GEOMETRY

## 3 · Constellation

180 satellites, 9 planes × 20, 53° inclination, 550 km. A 1/8-scale model of Starlink’s authorised Gen1 shell 1 (1,584 satellites, 72 × 22, 550 km / 53.0°, FCC 21-48, April 2021). The reduction is a rendering budget: 1,584 animated meshes will not hold 60 fps in a browser.

Four orbital data centres at 640 km, distinguished from relays by having compute.

Propagation. Real Starlink two-line elements from CelesTrak, propagated with SGP4 via satellite.js, converted ECI → geodetic through Greenwich Mean Sidereal Time. Fallback when the network is unavailable is an analytic Walker-Delta model:

lat = asin( sin(i) · sin(ν) )
Δlon = atan2( cos(i)·sin(ν), cos(ν) )
lon = (Ω + Δlon) mod 360 − 180

where i = inclination, ν = true anomaly advancing at 360° / 90 min, Ω = RAAN advancing at Earth-rotation rate.

## 4 · Path geometry

Great-circle distance by the haversine formula:

a = sin²(Δφ/2) + cos φ₁ · cos φ₂ · sin²(Δλ/2)
d = 2 R asin( √a )                                   R = 6,371 km

Slant range ground → satellite: √(d² + h²). Orbital arc between two satellites, measured at orbital radius: d · (R + h) / R.

Complete request path:

city →(slant, 550 km)→ uplink satellite
     →(orbital arcs)→ relay hops → orbital DC (640 km)
     →(orbital arc)→ satellite above the gateway
     →(slant)→ ground gateway

Two mistakes were made and fixed here, both material:

The downlink used to be a straight line through the planet. If the DC sat on the far side, “slant range DC → gateway” cut through the Earth. Fixed by relaying to a satellite actually above the gateway.

The round trip was counted twice. The chain already runs out to the DC and back down, so it is the complete request-plus-response path. Doubling it for RTT charged Sydney 45,000 km.

## 5 · Terrestrial fibre baseline — measured, not modelled

n_silica = 1.468        c_fibre = 204,190 km/s
route winding = 1.42    switching = 1.0 ms per 1,000 km
last-mile access = 5 ms each way

But the shipped baseline is measured, not this model. WonderNetwork ICMP RTTs between VPS nodes, cross-checked against Azure’s published 30-day P50 backbone medians. Datacentre-to-datacentre, therefore excluding consumer last-mile — a conservative baseline that understates the orbital advantage.

| Origin | km to nearest region | Measured | Azure P50 | Model |
| --- | --- | --- | --- | --- |
| New York | 325 | 7.5 ms | 8 | 15 |
| London | 637 | 14.4 ms | 17 | 21 |
| Delhi | 4,145 | 92.1 ms | 53 | 79 |
| Dubai | 4,836 | 122.2 ms | 100 | 91 |
| Lagos | 4,873 | 118.5 ms | — | 92 |
| Tokyo | 5,322 | 66.4 ms | 72 | 99 |
| Sydney | 6,305 | 92.7 ms | 95 | 116 |
| São Paulo | 7,626 | 113.1 ms | 118 | 138 |

Independent validation. Published crossover for a 550 km shell is 4,472 km (Chaudhry & Yanikomeroglu, arXiv:2203.00154); Handley (HotNets ’18) finds ~3,000 km for a higher shell. Using measured baselines this simulator loses at London (637 km) and New York (325 km) and wins from Delhi (4,145 km) upward — the same crossover region, arrived at independently.

# PART III · RAIN FADE — THE ITU CHAIN

This is the part most likely to be probed, so it is a standards implementation rather than an approximation, and it is unit-tested.

## 6 · ITU-R P.838-3 — specific attenuation

γ_R = k · R^α          dB/km,  R in mm/h

Coefficients are frequency-dependent, fitted as a sum of Gaussians in log-frequency:

log₁₀ k = Σⱼ aⱼ · exp( −[ (log₁₀ f − bⱼ) / cⱼ ]² ) + m_k · log₁₀ f + c_k     (4 terms)

      α = Σⱼ aⱼ · exp( −[ (log₁₀ f − bⱼ) / cⱼ ]² ) + m_α · log₁₀ f + c_α     (5 terms)

Note the asymmetry: k is computed in log space and exponentiated; α is not. Getting that backwards silently produces plausible-looking but wrong numbers.

Polarisation and geometry combine the horizontal and vertical coefficients:

k = [ k_H + k_V + (k_H − k_V) · cos²θ · cos 2τ ] / 2

α = [ k_H α_H + k_V α_V + (k_H α_H − k_V α_V) · cos²θ · cos 2τ ] / (2k)

θ = path elevation, τ = polarisation tilt (45° for circular).

Validation — npm run test:itu. Reproduces every published Table 5 value:

| f (GHz) | k_H | α_H | k_V | α_V |
| --- | --- | --- | --- | --- |
| 1 | 0.0000259 | 0.9691 | 0.0000308 | 0.8592 |
| 12 | 0.02386 | 1.1825 | 0.02455 | 1.1216 |
| 20 | 0.09164 | 1.0568 | 0.09611 | 0.9847 |
| 30 | 0.2403 | 0.9485 | 0.2291 | 0.9129 |

All 28 checks pass within 2%. This test is the credibility anchor: a mistyped constant cannot pass silently.

And it is the evidence for a design rejection. k at 1 GHz is 0.0000259 against 0.0916 at 20 GHz — rain barely touches UHF. Any dataset of ~437 MHz LoRa packets physically cannot contain Ka-band rain-fade signal.

## 7 · ITU-R P.839-4 — rain height

h_R = h₀ + 0.36 km

h₀ is the mean annual 0 °C isotherm height. P.839-4 ships it as a digital map; we use the latitude approximation h₀ = 5.0 − 0.075·(|φ| − 23) for |φ| ≥ 23°, else 5.0 km — stated as an approximation, not the current map.

## 8 · ITU-R P.618-13 — slant path, steps 1–10

Step 2   L_s = (h_R − h_s) / sin θ                                  θ ≥ 5°
Step 3   L_G = L_s · cos θ
Step 5   γ_R = k · R₀.₀₁^α
Step 6   r₀.₀₁ = 1 / [ 1 + 0.78·√(L_G γ_R / f) − 0.38·(1 − e^(−2 L_G)) ]
Step 7   ζ = atan[ (h_R − h_s) / (L_G r₀.₀₁) ]
         L_R = L_G r₀.₀₁ / cos θ   if ζ > θ,  else (h_R − h_s)/sin θ
         χ  = 36 − |φ|  if |φ| < 36°, else 0
         ν₀.₀₁ = 1 / { 1 + √(sin θ) · [ 31(1 − e^(−θ/(1+χ)))·√(L_R γ_R)/f² − 0.45 ] }
Step 8   L_E = L_R · ν₀.₀₁
Step 9   A₀.₀₁ = γ_R · L_E
Step 10  A_p = A₀.₀₁ · (p/0.01)^−[0.655 + 0.033 ln p − 0.045 ln A₀.₀₁ − β(1−p) sin θ]

Worked results at 20 GHz, 35° elevation, circular:

| Site | R₀.₀₁ | A₀.₀₁ | L_E | γ_R | h_R |
| --- | --- | --- | --- | --- | --- |
| Singapore | 120 mm/h | 54.4 dB | 4.4 km | 12.39 dB/km | 5.36 km |
| Lagos | 95 | 46.5 dB | 4.8 km | 9.76 | 5.36 |
| Sydney | 50 | 26.1 dB | 5.1 km | 5.07 | 4.56 |
| Frankfurt | 30 | 15.5 dB | 5.2 km | 3.01 | 3.31 |

Sanity check that matters: 54 dB at Singapore is why tropical Ka-band gateways use site diversity — you cannot close that much margin on one antenna.

## 9 · Instantaneous fade and outage

For a rain rate happening now rather than an annual statistic, the same specific attenuation and horizontal reduction are applied to the current rate:

| R (mm/h) | 0 | 1 | 5 | 10 | 20 | 30 | 50 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Fade (dB) | 0 | 0.88 | 4.53 | 8.36 | 14.31 | 19.32 | 27.78 |

Important distinction: γ_R = k·R^α is valid for any rain rate, but the P.618 statistics assume 1-minute integration. Our observations are hourly reanalysis, which smooths peaks — so instantaneous fade derived from them understates reality. Conservative in the direction that matters.

Outage probability maps residual margin through a logistic rather than a hard threshold, because fade estimated from hourly means is uncertain and real links degrade gracefully:

P(outage) = 1 / (1 + exp[ (M − A) / s ])        M = 6 dB margin, s = 1.0 dB

| Fade | 0 | 2 | 3 | 4 | 5 | 6 | 8 | 12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| P(outage) | 0.003 | 0.018 | 0.047 | 0.119 | 0.269 | 0.500 | 0.881 | 0.998 |

# PART IV · THE ROUTING ENGINE

## 10 · Cost in a single currency

Every term is in latency-equivalent milliseconds, so policy weights trade like against like.

C(DC) = w_lat · reach_ms(city, DC)
      + w_sol · 25 · eclipsed
      + w_rad · 40 · in_SAA
      + w_lat · 30 · γ · learned_tail_latency
      + w_rad · 30 · γ · learned_SAA_exposure

C(GW) = w_lat · reach_ms(city, GW)
      + w_wx  · fade_ms(weather)                       0 clear · 8 cloudy · 22 rain
      + w_wx  · 30 · γ · learned_rain_penalty
      + w_wx  · P(failure) · 900                       ← predictive term

reach_ms(a, b) = haversine(a, b) / 299,792 × 1000 × 2
γ (ADAPT_GAIN) = 0.6      FAILURE_COST_MS = 900

Why the unit discipline is load-bearing: an earlier version mixed normalised distance with binary 0/1 penalties. A single eclipse flag then outweighed half a planet of detour, and balanced routed a Delhi request to the far side of the Earth to find sunlight. Expressing eclipse as “worth 25 ms of detour at full weight” makes the trade explicit and auditable.

## 11 · Policy Pareto frontier — emergent, not designed

| Policy | w_lat / w_sol / w_rad / w_wx | p50 RTT | Rain exposure |
| --- | --- | --- | --- |
| latency | 0.95 / 0.05 / 0.05 / 0.05 | 61 ms | 13.7% |
| balanced | 0.50 / 0.50 / 0.50 / 0.40 | 67 ms | 13.9% |
| reliable | 0.20 / 0.30 / 0.95 / 0.90 | 73 ms | 0.0% |
| green | 0.20 / 0.90 / 0.20 / 0.30 | 78 ms | 5.5% |

reliable reaching exactly zero rain exposure for a 12 ms median cost was not specified — it is what a 0.90 weather weight does.

## 12 · Adaptive layer

src/telemetry.js — append-only event log in localStorage, capped at 6,000 records, quota-exceeded recovery. Windowed aggregation over 1H / 24H / 7D / 30D / ALL.

What is learned, and the principle behind the choice:

| Signal | Why it is real information |
| --- | --- |
| Gateway rain penalty | Weather is unpredictable from orbit; history is the only source |
| DC tail-latency penalty | Persistent geometric disadvantage no snapshot reveals |
| DC SAA exposure | Accumulated dose, weighted by the radiation coefficient |
| Best policy per origin | Surfaced as a recommendation, deliberately not auto-applied |

Deliberately NOT learned: historical eclipse share. The engine already reads dc.eclipsed instantaneously, so a windowed average is strictly less information than the flag one line above — and charging both double-counts the same orbital geometry.

The general test: if a system “learns” something it can already observe directly, the learning is decorative.

Measured: window selection changes routing in 4/32 (7D vs 30D) and 5/32 (24H vs 7D); adaptive on/off flips 6/32 routes; mean RTT 115 ms vs 118 ms.

# PART V · PREDICTIVE LAYER

## 13 · Weather — real observations, replayed

NASA POWER hourly API, parameter PRECTOTCORR, eight real teleport sites, 2,928 hours each (Apr–Jul 2026), baked to 141 KB of static JSON.

Unit trap, load-bearing: the hourly endpoint reports PRECTOTCORR in mm/day despite hourly sampling. It must be divided by 24. Missing this inflates every rain rate 24× and makes every fade figure nonsense.

| Gateway | Real site | Wet hours | Max |
| --- | --- | --- | --- |
| Singapore | Seletar Teleport | 91.1% | 31.28 mm/h |
| Lagos GW | Lekki | 68.1% | 9.33 |
| Mumbai | approximate — no published site | 31.7% | 11.26 |
| Tokyo GW | Hitachinaka | 28.8% | 15.06 |
| Virginia | Boydton, VA | 19.0% | 5.35 |
| Frankfurt | Usingen | 15.1% | 5.12 |
| São Paulo GW | Santana de Parnaíba | 11.9% | 4.04 |
| Sydney GW | Belrose (Optus) | 11.0% | 1.74 |

Why replay and not a random process: if the forecaster and the weather came from the same generator, a calibration score would only measure how well the model predicts its own random number generator.

Bound on what any weather-aware routing can achieve, measured directly: only Singapore produces fade above the 6 dB margin — 19% of hours, max 19.9 dB. Mumbai, Tokyo and Lagos have a handful; Frankfurt, Virginia, São Paulo and Sydney never.

## 14 · The single learned model

Target: P( fade > 3 dB at any point within the next H hours ), H ∈ {1, 3, 6, 12}, at 20 GHz / 35° elevation.

21 features, all strictly backward-looking:

rain, rain_lag{1,2,3,6,12}, trend_1h, trend_3h,
roll_mean_{3,6,24}, roll_max_{6,24}, wet_frac_24,
fade_now, hours_since_wet,
site_wet_fraction, site_mean_rain, abs_lat,
hour_of_day, doy_frac

Leakage discipline. Split by time, never randomly — consecutive hours are heavily correlated. Train hours 0–1,756 · validation 1,756–2,196 · test 2,196–2,928. A second split holds out entire gateways (Sydney, Lagos) to test generalisation to a site never seen.

Model: LightGBM, 90 estimators, 12 leaves, depth 5. Tree counts after early stopping: 86 / 57 / 78 / 71. Exported to a minimal node form — [feature, threshold, default_left, left, right] for splits, a bare number for leaves — 71 KB, roughly 15× smaller than LightGBM’s own dump.

Held-out results:

| Horizon | Climatology AUC | Persistence AUC / Brier | Logistic AUC / Brier | LightGBM AUC / Brier | Unseen-site AUC |
| --- | --- | --- | --- | --- | --- |
| +1h | 0.500 | 0.998 / 0.0294 | 0.998 / 0.0050 | 0.999 / 0.0030 | 0.999 |
| +3h | 0.500 | 0.990 / 0.0290 | 0.959 / 0.0101 | 0.991 / 0.0071 | 0.994 |
| +6h | 0.500 | 0.973 / 0.0307 | 0.840 / 0.0212 | 0.949 / 0.0139 | 0.987 |
| +12h | 0.500 | 0.944 / 0.0329 | 0.682 / 0.0542 | 0.892 / 0.0271 | 0.958 |

The honest reading: persistence is a very strong baseline and beats the model on ranking at 6h and 12h. What the learned model wins is calibration — Brier improves 2–10× at every horizon. That is the property a router needs, because it multiplies the probability by a cost rather than sorting by it.

## 15 · In-browser inference

Gradient-boosted trees evaluate trivially in JavaScript:

raw = Σ_t evalTree(t, x)          P = 1 / (1 + e^−raw)

Missing values follow each split’s stored default direction, matching training. No backend, no cold start, no CORS, works offline.

## 16 · Predictive cost term

expected = Σ_h w_h · P_h / Σ_h w_h            w = {1h:1.0, 3h:0.55, 6h:0.25, 12h:0.1}
C_pred   = 900 · [ P_now + confidence · expected · (1 − P_now) ]

The present is certain; only the forecast is damped by confidence. Horizon weights taper because a 12-hour prediction deserves less influence than a 1-hour one.

Confidence falls when conditions sit outside the training range — an injected 40 mm/h cell at a site whose recorded maximum is 2 mm/h is exactly where a confident prediction would be unearned. Below 0.6 the agent may only recommend. Measured: 90 mm/h at a site with an 11 mm/h maximum drops confidence to 0.45.

Fallback is mandatory. If prediction throws, is disabled, or returns nothing, the deterministic engine is unchanged. ML is never a single point of failure for routing.

# PART VI · THE AGENT

## 17 · Loop and structure

OBSERVE → PREDICT → PLAN → ACT → VERIFY → LEARN

A deterministic policy engine, not a language model. Tick every 4 s, decoupled from the 60 fps render loop.

Tools: inspect_network, predict_gateway_risk, evaluate_route, plan, step, verify, log_decision.

## 18 · Anti-thrash gates

Without these an agent oscillates between two gateways and makes the network worse than leaving it alone.

MIN_HOLD_MS          20,000    route must be in place this long
COOLDOWN_MS          15,000    wait after acting
MIN_IMPROVEMENT_MS       25    expected cost improvement required
MIN_RISK_DROP          0.15    predicted risk must fall by this much
ACTION_COST_MS           12    moving is not free
GATEWAY_RISK_TRIGGER   0.45    wakes the agent
MIN_CONFIDENCE_TO_ACT   0.60   below this, recommend only

Verified behaviour: healthy network → HOLD / current_route_is_best; immediate move → HOLD / min_hold_time; 90 mm/h out-of-distribution → confidence 0.45, autonomy blocked.

## 19 · Modes and verification

OFF (recommend nothing) · ASSIST (propose, wait for approval — the default) · AUTOPILOT (act).

Nobody flies an autonomous burn on an AI recommendation today, and the same caution applies here.

Every decision is scored against what the weather actually did, producing a Brier score and a reliability table. A prediction that is never checked is a claim, not a forecast.

Live scenario, verified end to end:

Delhi served via Mumbai (10 ms)
  → rain cell injected at Mumbai
  → predicted risk 0.938
  → REROUTE Mumbai → Tokyo GW, +334 ms expected cost saved, risk −0.937, confidence 0.90
  → VERIFIED: fade actually reached 19.32 dB at the abandoned gateway

# PART VII · THE EXPERIMENT

## 20 · Design

Paired, four arms. All arms see the identical request schedule against the identical weather. A request fails when fade at its gateway exceeds the 6 dB margin — the same physical rule everywhere, so the difference is the routing, not the luck.

| Arm | Vision |
| --- | --- |
| A CURRENT | static weather label only — the pre-predictive engine |
| R REACTIVE | current fade, no forecast |
| B PREDICTIVE | current fade plus forecast |
| C AUTOPILOT | B, plus a per-origin sticky route the agent may move under its gates |

Separating REACTIVE from PREDICTIVE is the point — it isolates the value of predicting from the value of merely observing, which is the question a reviewer will actually ask.

## 21 · Results — 5,600 requests over 2,800 recorded hours

| Arm | Failures | Rate | Mean RTT | p95 RTT | Reroutes | Proactive |
| --- | --- | --- | --- | --- | --- | --- |
| A · CURRENT | 80 | 1.43% | 4.0 ms | 12.7 ms | — | — |
| R · REACTIVE | 2 | 0.04% | 3.7 ms | 10.0 ms | — | — |
| B · PREDICTIVE | 0 | 0.00% | 3.9 ms | 11.6 ms | — | — |
| C · AUTOPILOT | 0 | 0.00% | 9.2 ms | 31.9 ms | 9 | 7 (78%) |

Finding 1 — most of the benefit is from observing, not forecasting. Reacting to current fade removes 78 of 80 failures. Forecasting removes the last 2, for +0.2 ms mean latency. Real, but a far smaller marginal gain than a predictive system is usually sold on.

Finding 2 — a negative result about the agent. Arm C reaches the same zero failures as plain predictive routing while costing +5.3 ms mean latency, because a sticky route with hysteresis stays on a suboptimal gateway between moves. Here, re-selecting every request is free, so hysteresis buys nothing. Its value belongs to a case this model does not represent — session affinity, cache warmth, signalling cost.

Both findings are shown in the product, not buried.

# PART VIII · DEFECTS FOUND AND FIXED

Every one was verified before being fixed.

| # | Defect | Consequence |
| --- | --- | --- |
| 1 | sleep() stored only the timer id; clearTimeout killed the resolver | Spacebar or ⏭ permanently bricked the entire UI. Most likely why round 1 failed |
| 2 | await requestAnimationFrame unraced | App silently never booted in a background tab |
| 3 | RTT was 6000 + hops×1200 | All 8 cities returned identical latency — in a CDN |
| 4 | Policy comparison used hardcoded ×1.15 / ×1.22 | Named reliable slowest when it was fastest |
| 5 | Cost function mixed normalised distance with binary flags | balanced routed Delhi to the far side of Earth |
| 6 | Round-trip path doubled | Sydney charged 45,000 km |
| 7 | Insights page claimed 72 sats / 6 planes and two nonexistent cost terms | Documentation contradicted the code |
| 8 | Deep-dive tables scored with a retired cost function | Ranked a different winner than the router chose, in 5/32 cases |
| 9 | Gateway weather was a frozen literal | Learned penalty collapsed to weatherMs/22 — an obfuscated constant |
| 10 | Learned radiation scaled by the solar weight | reliable discounted it 3×, green amplified it 4.5× |
| 11 | Seed used a different cost function from the router | Disagreed on 17.7% of balanced DC picks |
| 12 | SAA box spanned −80..+10 longitude | Clipped 40° of the published extent |
| 13 | predictiveCostMs returned 0 with no horizons | Made the REACTIVE control arm identical to CURRENT — would have shipped as a false finding |
| 14 | Experiment used one global sticky gateway for 8 origins | Agent arm never rerouted |
| 15 | Agent tick re-rendered the overlay | Wiped experiment results |
| 16 | Autopilot panel overlapped the control column | Hid the policy weight bars |

# PART IX · WHAT WAS REJECTED, AND WHY

| Rejected | Evidence |
| --- | --- |
| Train on TinyGS for Ka-band rain degradation | TinyGS is LoRa at ~437 MHz. ITU-R P.838-3: k = 0.0000259 at 1 GHz vs 0.0916 at 20 GHz. The dataset cannot contain the signal |
| ML for rain fade | Solved analytically by P.618/838/839; claiming ML invites an unanswerable question |
| FastAPI backend | Would be the demo’s only runtime dependency; 71 KB of JSON runs in-browser |
| Full autonomy claim | Reframed as decision support with a human gate |
| SatNOGS / Space-Track adapters | Not needed once fade became analytic |
| IMERG pipeline | Requires Earthdata auth; POWER is open and sufficient |
| Historical-TLE reconstruction | Out of scope for gateway weather |
| LSTM / TCN | Not attempted — GBM was not yet exhausted |
| Spacecraft anomaly model | Phase 2 |
| LLM explanation layer | Structured decision factors are clearer and cannot hallucinate |

# PART X · LIMITATIONS

Stated because being caught at them is worse than declaring them.

- The 30-day request history is simulated and labelled SIMULATED in the UI with its deterministic seed. Grounding fixes the baseline it is compared against, not the traffic itself.

- The DC feedback loop is weak — averaged over days, every DC is eclipsed roughly equally, so there is little to learn. Reported as-is rather than inflated by raising a gain.

- The constellation is 1/8 scale.

- The SAA is a rectangle over an oval that has split into two lobes and drifts west ~0.3°/yr.

- Maximum ISL range (5,400 km) is not modelled — a real omission at this scale.

- Rain rates are hourly reanalysis, which smooths the 1-minute peaks P.618 statistics assume. Conservative.

- Only one gateway in the trace produces fade above margin, which bounds the measurable benefit.

- Mumbai’s coordinate is approximate — no published gateway location exists.

# APPENDIX · FILES AND COMMANDS

src/predict/itu.js          204   ITU-R P.838-3 / P.618-13 / P.839-4
src/predict/weather.js      200   trace clock, forecasting, injection
src/predict/agent.js        310   policy engine, gates, verification, calibration
src/predict/experiment.js   228   paired four-arm experiment
src/predict/ui.js           366   autopilot panel and overlay
src/predict/gbm.js           41   tree-ensemble evaluator
src/predict/config.js        36   all tunable constants
ml/fetch_weather.py          87   NASA POWER ingestion
ml/train_fade_risk.py       276   training, evaluation, JSON export

npm run test:itu        validate physics against published ITU tables
npm run data:weather    re-fetch NASA POWER traces
npm run train:fade      retrain and re-export the forecaster
npm run dev / build

Data sources: NASA POWER hourly API · CelesTrak · WonderNetwork · Microsoft Azure latency statistics · ITU-R P.618-13 / P.838-3 / P.839-4 / P.837 · FCC 21-48 · Chaudhry & Yanikomeroglu arXiv:2203.00154 · Handley, HotNets ’18.

Every figure in this document was measured from the running system or transcribed from a cited source.